## ESP32-P4 Peripheral Interface Connections – UK Demo

This section outlines the hardware interface connections used in the UK demonstration setup based on the ESP32-P4 controller. The ESP32-P4 serves as the central processing unit, interfacing with multiple peripherals including dual OLED displays and a touch sensor. The connection tables provided below define the GPIO assignments, power distribution, and communication interfaces required for reliable operation of the demonstration platform. These connections have been selected to ensure efficient communication between the ESP32-P4 and the connected peripherals while maintaining a simple and scalable hardware architecture.

1. Dual SPI OLED Display Connections:

| ESP32-P4 | Display 1 | Display 2 |
| --- | --- | --- |
| GPIO 23 | CLK | CLK |
| GPIO 22 | DIN | DIN |
| GPIO 21 | DC | DC |
| GPIO 20 | RST | RST |
| GPIO 26 | CS | — |
| GPIO 27 | — | CS |
| 3V3 | VCC | VCC |
| GND | GND | GND |

2. Touch Sensor Connections:

| ESP32-P4 | Touch Sensor |
| --- | --- |
| 3V3 | 3.3V (VCC) |
| GND | GND |
| GPIO 7 | I2C SDA |
| GPIO 8 | I2C SCL |

3. Other Peripheral Connections